## MSE 23000 Homework Assignment – Week 3 (10 pts)

Mechanical Behavior

Over the next week, think about the following problems and provide your responses as prompted below using this document as a template. Responses can be typed and/or inserted as scans or images (e.g., of hand-written calculations or sketches). These questions are reflective of typical exam questions – hence, the students who complete their homework assignments are the best prepared for their upcoming exams. Additionally, you are encouraged to attempt all homework problems before attending your weekly recitation so that you are prepared to ask questions and seek guidance & hints from your recitation instructor. This assignment is worth 10 points and will be graded for completion only. Submit your assignment as a single PDF document by using the appropriate link on Brightspace. You may choose to complete this assignment independently or work in groups but all submitted responses must be unique/your own work. Correct responses will be discussed in your next recitation lecture.

(1 pt) 1. Callister 6.4: A cylindrical specimen of a titanium alloy having an elastic modulus of 107 GPa and an original diameter of 3.8 mm will experience only elastic deformation when a tensile load of 2,000 N is applied. Compute the maximum length of the specimen before deformation if the maximum allowable elongation is 0.42 mm.

L=42*(1.13*10-5)*(107*109)2000

L = 2.28m

(2 pts) 2. Callister 6.7: For a bronze alloy, the stress at which plastic deformation begins is 275 MPa and the modulus of elasticity is 115 GPa. (1 pt each)

- What is the maximum load that may be applied to a specimen with a cross-sectional area of 325 mm2 without plastic deformation occurring?

F= 275*106*325*10-6

F=89.375 N

- If the original specimen length is 115 mm, what is the maximum length to which it may be stretched without causing plastic deformation?

L=275*106115*106

114.725mm

(2 pts) 3. Callister 6.23: A cylindrical rod 100 mm long and having a diameter of 10.0 mm is to be deformed using a tensile load of 27,500 N. It must not experience either plastic deformation or a diameter reduction of more than 7.5 x 10-3 mm. Of the materials listed in the following table, which are possible candidates and why?

| Material | E (GPa) | Yield Strength (MPa) | Poisson’s Ratio |
| --- | --- | --- | --- |
| Aluminum alloy | 70 | 200 | 0.33 |
| Brass alloy | 101 | 300 | 0.34 |
| Steel alloy | 207 | 400 | 0.30 |
| Titanium alloy | 107 | 650 | 0.34 |

All of the materials appear to experience plastic deformation and diameter reduction

(5 pts) 4. Callister 6.29: A cylindrical specimen of aluminum having a diameter of 12.8 mm and a gauge length of 50.800 mm is pulled in tension. Use the load-elongation data in the following table to compute parts (a) through (e). (1 pt each)

| Load (N) | Length (mm) |
| --- | --- |
| 0 | 50.800 |
| 7330 | 50.851 |
| 15100 | 50.902 |
| 23100 | 50.952 |
| 30400 | 51.003 |
| 34400 | 51.054 |
| 38400 | 51.308 |
| 41300 | 51.816 |
| 44800 | 52.832 |
| 46200 | 53.848 |
| 47300 | 54.864 |
| 47500 | 55.880 |
| 46100 | 56.896 |
| 44800 | 57.658 |
| 42600 | 58.420 |
| 36400 | 59.182 |
| Fracture! (boom) |  |

- Use a computer to create a scatter plot of all the data in the above table as engineering stress vs. engineering strain.

- Compute the modulus of elasticity.

E=π(12.84)2

E=32.169

- Determine the yield strength using the “0.2% strain offset” method. Hint: it might be easiest to do this using a “zoomed-in” plot of only the small-strain data, e.g. from 0 to 0.010.

50000

- Determine the tensile strength of this alloy. 50000

- What is approximate ductility, in percent elongation? 50